| Report | Comments on QAQC | Discussion (Note 1) |
| --- | --- | --- |
| Phase 2 Site Investigation | 2 ARL laboratory reports did not contain NATA certification stamps. | The samples analysed in these reports do not appear in results tables, and it has been assumed that the assessor did not use them in site characterisation. |
|  | Laboratory report PES1124 was not provided. |  |
|  | Secondary laboratory reports for groundwater analysis were not provided. |  |
|  | A large number of blinds and splits had RPDs that exceeded the specified range. | These are likely to be associated with; Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Soil samples being collected from heterogeneous fill material, or Fissures and cracks in the fill providing preferential pathways that lead to an uneven distribution of TPH. |
|  | QC sampling frequency for soil field splits and blinds were less than the number recommended in the Australian Standard |  |
|  | 52 laboratory duplicates had RPDs that exceeded specified range and 9 matrix spikes and a small number of secondary laboratory spikes had poor recoveries. | The matrix spikes with poor recoveries had satisfactory RPDs and were for analytes for which sample results were all non-detects. |
|  | Internal Laboratory QA was not reported for the Asbestos Soil and Air monitoring analysis |  |
|  | In 15 analyte suites all samples clearly exceeded the recommended holding times and 10 had some samples exceeding the holding times. (note 2) | The majority of analytes that clearly or partly exceeded the hold times were for TPH (semi volatile or volatile/BTEX) but some PAHs and pesticides were also included. |
|  | No chain of custody was provided for ALS Laboratory Report PES1093. |  |
|  | No calibration certificates were provided for Field Instruments. | URS standard investigation Methodology for environmental sampling describes PID calibration against 50ppm for benzene, however the assessor did not provide calibration certificates |
|  | Laboratory detection limits were above guidelines for Endrin and Chloropyrifos (Aquatic Ecosystem guidelines for both were 0.008 ug/L and detection limits were 0.5 ug/L) | The site history does not indicate that these are contaminants of concern on the Waterfront Development site. |
| Phase 3/4 Site Investigation | No laboratory reports were provided for the analysis of; FHSB12A; SBMW26_5.5; SBMW26_4.0, WATP50_4.3, FHSB15, SHTP11, SPSB04, RRMW16, FHMW01, SHTP12, MW17, KDMW02, FHMW02, SHMW8, BPMW44, KDMW03, WAMW07, SPSB02, BPMW45A, KDMW04, WAMW32, SPSB03, BPMW45B, KDMW05. |  |
|  | A large number of blinds and splits had RPDs that exceeded the specified range. | These are likely to be associated with; Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Soil samples being collected from heterogeneous fill material, or Fissures and cracks in the fill providing preferential pathways that lead to an uneven distribution of TPH and localised variations in concentrations within samples. |
|  | QC sampling frequency for soil field splits and blinds were less than the number recommended in the Australian Standard. |  |
|  | A number of the results for laboratory duplicates and matrix spikes were outside the recommended range. | The matrix spikes with poor recoveries were for analytes for which sample results were all non-detects. The high RPDs for laboratory duplicates can be explained as stated previously except for some high groundwater RPDs for K, Zn and NH3. |
|  | The secondary laboratory, reported one internal spike with low recovery for metals (69-80%). | This can be attributed to a particularly low recovery for Ni. |
|  | Very small traces of Zinc (<0.008 mg/L) were found in 12 of the Rinsate Blanks for Phase 4 |  |
|  | In one laboratory batch the recommended hold time for volatiles in some soil samples was exceeded. In 3 secondary laboratory batches, (017246,017247,017406) the recommended hold times were exceeded for TPH, PAH, metals and Volatiles. |  |
|  | Chain of custodies were not provided for ALS Reports ES1058 and AES50116. |  |
|  | No calibration certificates were provided for Field Instruments. | URS standard investigation methodology for environmental sampling describes PID calibration against 50 ppm benzene, however URS did not provide calibration certificates. |
|  | Field procedures were not provided for sample preservation, storage and minimisation of volatile losses. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality. |
|  | Laboratory detection limits were above guidelines for Endrin and Chloropyrifos (Aquatic Ecosystem guidelines for both were 0.008 ug/L and detection limits were 0.5 ug/L) | The site history and testing program indicates that these are not contaminants of concern at the on the Waterfront Development site. |
| Pre-Remediation | Some blinds and splits had RPDs that exceeded the specified range. | These are likely to be associated with; Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Soil samples being collected from heterogeneous fill material, or Fissures and cracks in the fill providing preferential pathways that lead to an uneven distribution of contaminants. |
|  | All chain of custodies for ALS reports were not provided. |  |
|  | Hold times could not be verified from chain of custodies. |  |
|  | No calibration certificates were provided for Field Instruments. | URS standard investigation methodology for environmental sampling describes PID calibration against 50 ppm benzene, however URS did not provide calibration certificates |
|  | The assessor’s field procedures were not provided. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality. |
| GW 04-05 | Laboratory reports were not provided for many samples, including the majority of the September 04 samples and select samples from other monitoring rounds. |  |
|  | A total of 27 blinds and splits had been sampled over the 4 monitoring rounds. 55 analyte results within the blind and split samples had RPDs that exceeded the specified range (<30% inorganics; <50% organics). | Of the 55 analyte results, 32 of the exceedences can be attributed to: Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Trace levels of separate phase hydrocarbons The remainder of exceedences (23 results) were associated with: sulphate, nitrite + nitrate, metals (Hg, Cr, Zn, Cd, Mn), PAHs (fluoranthene, phenanthrene, pyrene) halogenated aliphatics (trichloroethene, trans – 1,2-dichloroethene) and physico-chemical parameters (total bicarbonate, TDS) Noting the heterogeneous nature of the materials sampled and the extensive range of RPDs calculated, the auditor considers that the exceedences are typical of what might be expected and do not preclude the use of the results for characterisation of the presence of contamination across the Waterfront Development site. |
|  | QC sampling frequency for blind duplicates were less than the number recommended in the Australian Standard for 3 out of the 4 sampling rounds (December 04 – 0 of 44), February 05 (0 of 67) and May 05 (0 of 66)). |  |
|  | A total of 5 laboratory duplicate samples had RPDs that exceeded the recommended levels. (RPDs - 30% inorganics and 50% organics). Ammonia had a RPD of 196% (Primary – 1.06 mg/kg ; Duplicate- <0.01 mg/kg). Internal Laboratory QC was not provided for the February sampling period | The exceedence of the criteria for 4 of the 5 results are likely to be associated with; Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Soil samples being collected from heterogeneous fill material, or Fissures and cracks in the fill providing preferential pathways that lead to an uneven distribution and localised variations in concentrations of TPH within samples. |
|  | 9 out of 18 laboratory batches had matrix spike results exceeding the recommended range of 70% to 130% for the phenol suite (Range: 34-58%) (Pentachlorophenol also had a low recovery for 1 of 9 batches). | Low phenol recoveries are commonly observed in laboratory analysis and these results are not considered to be of concern. |
|  | Rinsate Blanks Out of the 17 rinsate blanks measured for metals, low concentrations were found for: Cadmium (9 of 17 rinsates), chromium (1 of 17 rinsates), copper (2 of 17 rinsates), manganese (2 of 17 rinsates) and zinc (13 of 17 rinsates) 1 of 18 rinsate blanks measured for TPH/BTEX had a C10 – C36 concentration of 450 g/L (Feb 2005 Sampling round). All 15 rinsate blanks measured for chloroform returned values greater than the LOR. Trip Blanks Low concentrations of the following analytes were found in 4 of the 13 trip blanks measured for TPH/BTEX: Ethylbenzene (3 of 4), meta & para xylene (3 of 4) and C6-C9 (1 of 4). | In all cases the concentrations were below the ANZECC/ARMCANZ 2000 trigger values for the 95% protection of aquatic ecosystems. Three monitoring wells were sampled on this day (MW22, MW27A and MW44). Because TPH C10 – C36 concentrations were reported as less than the laboratory LOR for all three, it is therefore considered not to have affected the overall data quality. Chloroform concentrations were all below laboratory LOR for the groundwater sample results, therefore the rinsate results do not affect overall data quality. These results suggest that there might be some false positives in analysis of samples, but would not lead to a conclusion that contamination was not present. |
|  | A total of 26 out of the 50 analyte suites have exceeded the recommended holding times. 22 of the 26 analyte suites exceeded the 24 hour hold time: Methane (5 of 26), TPH (v)/BTEX (9 of 26), TPH (sv) (8 of 26), PAH/Phenols (1 of 26); failed holding time by 3 days 3 of the 26 had some samples within and some samples exceeding the hold times: pesticides, volatiles and PAH/phenols. | As sample preparation information was not included in the report, the exceedences are based on the assumption that hydrochloric acid was not used as a preservative. The practical difficulty in complying with short holding time requirements in samples taken in Darwin are noted. The potential for some loss of contaminant in some samples is noted and has been considered. |
|  | Chain of custodies were present and correctly completed for all Laboratory batches except ALS batch number ES50051. | Minor discrepancy; not considered to be of concern. |
|  | The equipment supplier’s calibration certificates were not included in the report |  |
|  | Field procedures were not provided for minimisation of volatile losses. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality |
|  | Laboratory detection limits were above guidelines for; Anthracene, Aquatic ecosystem guidelines are 0.4 ug/L and detection limits were 1.0 ug/L Benzo(a)pyrene, Aquatic ecosystem guidelines are 0.2 ug/L and detection limits were 0.5 ug/L. Endrin, Aquatic ecosystem guidelines are 0.008 ug/L and detection limits were 0.5 ug/L Chloropyrifos, Aquatic ecosystem guidelines are 0.008 ug/L and detection limits were 0.5 ug/L.. | Anthracene, Endrin and Chloropyrifos are not contaminants of concern across the Waterfront Development site. An isolated exceedence in benzo(a)pyrene was detected in MW09 in the Stokes Hill area during the Sept 2004 sampling round. Concentrations of benzo(a)pyrene were below the LOR in the following three sample rounds. |
| GW 05-06 | A total of 19 blinds and splits had been sampled over the 4 monitoring rounds. 58 analyte results within the blind and split samples had RPDs that exceeded the specified range (<30% inorganics; <50% organics). The remainder of exceedences are associated with iron (total, ferrous, ferric) and other physio-chemical parameters. | Of the 58 analyte results, 34 of the exceedences can be attributed to: Low analyte concentrations which exaggerate the RPD relating to small total concentration differences, or Trace levels of separate phase hydrocarbons Results recorded over a number of sampling rounds provide confidence in the overall body of results. |
|  | QC sampling frequency for blind duplicates were less than the number recommended in the Australian Standard for the 4 sampling rounds |  |
|  | A total of 20 laboratory duplicate samples had RPDs that exceeded the recommended levels. (RPDs - 30% inorganics and 50% organics) | In all of these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerate the RPD relating to small total concentration differences. |
|  | 5 out of 44 laboratory batches had matrix spike results exceeding the recommended range of 70% to 130%. 17 out of the 40 batches had matrix spike results that could not be determined by the lab. | Of the 5 batches, 3 batches had results that were still within the laboratory’s dynamic recovery limits. The other 2 batches had the following analytes which exceeded the criteria: Batch EP0601302: Pentachloroethane 133% (Lab Upper Control Limit – 126%) and Ammonia 57.7% (Lower Data Quality Objective 70%). Batch EP0601332: Ammonia 57.7% (Lower Data Quality Objective 70%). In these cases, the lab reported the background concentration in the analyte as too high in comparison with the spike concentration; therefore a spike recovery could not be determined. |
|  | The recommended 24 hr Holding Time for the following analytes has been exceeded: pH (16 of 44 laboratory batches in exceedence) Ferrous Iron (6 of 44 batches), Nitrite/Nitrate (5 of 44 batches) and Nitrite (1 of 44 batches) No data was provided regarding holding times for the secondary lab (Amdel). Amdel was responsible for 4 of the 44 laboratory batches carried out. | Field pH testing was also undertaken and is considered to be a more representative measure of groundwater conditions As analysis was undertaken outside of the recommended holding times, the results for these analyses may be biased slightly low. |
|  | The equipment supplier’s calibration certificates were not included in the report | URS standard investigation methodology for environmental sampling describes PID calibration against 50 ppm benzene, however the assessor did not provide calibration certificates |
|  | Field procedures were not provided for minimisation of volatile losses. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality |
|  | Laboratory detection limits were not less than or equal to the guidelines for: Anthracene, detection limits were 1.0 ug/L and aquatic ecosystem guidelines are 0.4 ug/L Benzo(a)pyrene, detection limits were 0.5 ug/L and aquatic ecosystem guidelines are 0.2 ug/L | Anthracene is not a contaminant of concern across the Waterfront Development site. Benzo(a)pyrene concentrations above the LOR were only previously detected from well MW09 (Sept 2004 sampling round) but this well has had six subsequent samples which have reported results below the LOR. |
| GW 07-08 | SGS 62134: The COC documentation is missing for laboratory sample IDs 1 to 10. |  |
|  | Several samples were received by the laboratory (2007: ALS primary laboratory, Amdel secondary laboratory; 2008: SGS primary laboratory, ALS secondary laboratory) above the recommended range of 4oC + or – 2oC. In some cases, the sample temperature upon receipt by the lab was not specified on the Chain of Custody documentation. | A random check of the sample temperatures upon receipt from the lab indicated the following exceedences: Report Number; Lab; Comment. 2007. ES0714975; ALS; Temperature recorded as 16.2oC (ice bricks were present).. ES0715178; ALS; Temperature recorded as 10.4oC (ice bricks were present).. ES0715041; ALS; Temperature recorded as 15.8oC (ice bricks were present).. ES0715104; ALS; Temperature recorded as 9.7oC (ice bricks were present).. ES0715338; ALS; Temperature recorded as 7.8oC (ice bricks were present).. 07ENCA0023017; Amdel; Temperature upon receipt from Amdel was not specified, samples first received by ALS as 16.2oC.. 07ENCA0023305; Amdel; Temperature was recorded as 10.0oC (ice present but not specified on COC if free ice or ice bricks).. 2008. 62134; SGS; Temperature recorded as 5.7oC (ice bricks were present).. 62397; SGS; Temperature recorded as 9.0oC (free ice was present).. ES0809374; ALS; Temperature recorded as 13.5oC (ice present but not specified on COC if free ice or ice bricks).. ES0809799; ALS; Temperature recorded as 9.4oC (cooling method not specified).. ES0809981; ALS; Temperature recorded as 4.4oC (cooling method not specified).. |
|  |  | The auditor notes that volatile losses may have occurred due to the increase in temperature. As a result, the associated VOCs may have been under-reported. |
|  | The preservation and storage method has not been specified for the samples having a delay between sample collection and sample relinquishment. | ES0714975: Samples relinquished on 29/10/2007 and received by lab on 30/10/2007; ES0715178: Samples relinquished on 01/11/2007 and received by lab on 02/11/2007; ES0715041: Samples relinquished on 30/10/2007 and received by lab on 31/10/2007; ES0715104: Samples relinquished on 31/10/2007 and received by lab on 01/11/2007; ES0715338: Samples relinquished on 05/11/2007 and received by lab on 06/11/2007; 07ENCA0023017: Samples relinquished on 29/10/2007 and received by lab on 31/10/2007; 07ENCA0023305: Samples relinquished on 01/11/2007 and received by lab on 02/11/2007; 62134: Samples relinquished on 01/07/2008 and received by lab on 02/07/2008; 62324: Samples relinquished on 08/07/2008 and received by lab on 09/07/2008; 62397: Samples relinquished on 10/07/2008 and received by lab on 11/07/2008; ES0809374: Samples relinquished on 01/07/2008 and received by lab on 02/07/2008; ES0809799: Samples relinquished on 08/07/2008 and received by lab on 09/07/2008; ES0809981: Samples relinquished on 10/07/2008 and received by lab on 11/07/2008. Although URS did not specify the preservation and storage methods employed, the URS quality assurance program for other investigations with the Stage 2a site, have included preservation and storage of samples upon collection and during transfer to the laboratory. |
|  | The recommended holding has been exceeded for several samples within the 2007 & 2008 laboratory reports. | The following holding time exceedences have been noted (Matrix: water): Lab Report; Analyte; Comment. ES0714975 (ALS); pH; All samples exceed HT by 1 day. ES0715178 (ALS); pH; All samples exceed HT by 1 day. ES0715041 (ALS); pH; All samples exceed HT by 1 day. ES0715104 (ALS); pH; All samples exceed HT by 1 day. ES0715338 (ALS); pH; All samples exceed HT by 1 day. 07ENCA0023017; Ammonia; QC02 exceeds HT by 1 ½ days. Nitrate; QC02 exceeds HT by 3 days. 07ENCA0023305; Alkalinity, Nitrate; QC04 exceeds HT by 3 days. pH, Ammonia; QC04 exceeds HT by 3 ½ days. Sulphide; QC04 exceeds HT by 4 days. 62134; TDS; Exceeds HT by 5 days: MW109, MW110, MW113, MW21A, WFQCB3, WFQCC3 Exceeds HT by 6 days: MW101, MW102, MW103, MW104, MW105, WFQCB2, WFQCC 2 Exceeds HT by 9 days: MW107, WFQCB1, WFQCC1. Alkalinity; Exceeds HT by 1 day: MW109, MW110, MW113, MW21A, WFQCB3, WFQCC3 Exceeds HT by 2 days: MW101, MW102, MW103, MW104, MW105, WFQCB2, WFQCC2 Exceeds HT by 5 days: MW107, WFQCB1, WFQCC1. HT: Holding Time. |
|  |  | Lab Report; Analyte; Comment. Sulphide; Exceeds HT by 3 days: MW101, MW102, MW103, MW104, MW105 Exceeds HT by 6 days: MW107 Exceeds HT by 2 days: MW109, MW110, MW113, MW21A, WFQCB3, WFQCC3. Nitrate; Exceeds HT by 6 days: MW101, MW102, MW103, MW104, MW105, WFQCB2, WFQCC2 Exceeds HT by 9 days: MW107, WFQCB1, WFQCC1 Exceeds HT by 5 days: MW109, MW110, MW113, MW21A, WFQCB3, WFQCC3. Nitrite; Exceeds HT by 7 days: MW101, MW102, MW103, MW104, MW105, WFQCB2, WFQCC2 Exceeds HT by 10 days: MW107, WFQCB1, WFQCC1 Exceeds HT by 6 days: MW109, MW110, MW113, MW21A, WFQCB3, WFQCC3. 62324; TDS; Exceeds HT by 7 days: MW09, MW10, MW120, MW123 Exceeds HT by 8 days: MW108, MW115, MW128, WFQC102. Alkalinity; Exceeds HT by 1 day: MW108, MW115, MW128, WFQC102. Sulphide; Exceeds HT by 6 days: MW09, MW10, MW120, MW123 Exceeds HT by 7 days: MW108, MW115, MW128, WFQC102. Nitrate; Exceeds HT by 2 days: MW09, MW10, MW120, MW123 Exceeds HT by 3 days: MW108, MW115, MW128, WFQC102. HT: Holding Time. |
|  |  | Lab Report; Analyte; Comment. Nitrite; Exceeds HT by 3 days: MW09, MW10, MW120, MW123 Exceeds HT by 4 days: MW108, MW115, MW128, WFQC102. 62397; TDS; Exceeds HT by 8 days: MW121, MW125, MW129, MW134, MW135, WFQC103, WFQCB8, WFQCC8, MW117. Alkalinity; Exceeds HT by 5 days: MW121, MW125, MW129, MW134, MW135, WFQC103, WFQCB8, WFQCC8, MW117. Sulphide; Exceeds HT by 9 days: MW121, MW125, MW129, MW134, MW135, WFQC103, WFQCB8, WFQCC8, MW117. Nitrate; Exceeds HT by 5 days: MW121, MW125, MW129, MW134, MW135, WFQC103, WFQCB8, WFQCC8, MW117. Nitrite; Exceeds HT by 6 days: MW121, MW125, MW129, MW134, MW135, WFQC103, WFQCB8, WFQCC8, MW117. ES0809799; Nitrite as N; Exceeds HT by 1 day: WFQC202. ES0809981; Nitrite as N; Exceeds HT by 3 days: WFQC203. Sulfite; Exceeds HT by 1 day: WFQC203. HT: Holding Time. |
|  | A total of 2 blinds (QC01 and QC03) and 2 splits (QC02 and QC04) had been sampled during the 2007 investigation. 9 analyte results within the blind and split samples had RPDs that exceeded the specified range. | The following analytes were found to exceed the specified RPD range: Sample ID; Analyte; Primary Result (mg/L); Duplicate Result (mg/L); RPD (%). QC01 (blind); Ionic Balance; 1.91; 0.7; 92.7. Zinc; 0.009; 0.006; 40.0. QC02 (split); TPH C6-C9; <20; 39; 64.4. Benzene; <5; 1.3; 117.5. Napthalene; 82; 140; 52.3. QC03 (blind); TPH C15-C28; 100; 200; 66.7. Ionic Balance; 0.98; 0.71; 32.0. QC04 (split); TPH C15-C28; 100; 4320; 191. TPH C29-C36; 50; 347; 149.6. In all of these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerated the RPDs relating to small total concentration differences. |
|  | A total of 3 blinds (WFQC 101, WFQC 102 and WFQC 103) and 2 splits (WFQC 202 and WFQC 203) had been sampled during the 2008 investigation. 16 analyte results within the blind and split samples had RPDs that exceeded the specified range. | The following analytes were found to exceed the specified RPD range: Sample ID; Analyte; Primary Result (mg/L); Duplicate Result (mg/L); RPD (%). WFQC 101 (blind); Sulphate; 40.0. Chromium; 88.9. Zinc; 46.6. WFQC 102 (blind); Chloride; 32.1. Sulphate; 85.7. Sulphur; 38.0. Manganese; 51.4. WFQC 202 (split); Calcium; 57.2. Chloride; 47.7. Magnesium; 53.0. Potassium; 48.3. Sodium; 77.0. Sulphate; 121.5. Sulphur; 138.1. WFQC 203 (split); TPH C10-C14; 137.3. Sulphate; 45.6. In all of these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerated the RPDs relating to small total concentration differences. |
|  | 2007 No rinsate blanks were collected and analysed during the 2007 investigation. No trip or field blanks were reported during this investigation 2008 Rinsate Blanks (RB): A total of 6 rinsate blanks (WFQC C1, WFQC C2, WFQC C3, WFQC C6, WFQC C7 and WFQC C8) were collected and analysed by SGS. Of these 6 rinsate blanks, 5 of the blanks were found to have concentrations above the LOR. Field Blanks (FB): A total of 6 field blanks (WFQC B1, WFQC B2, WFQC B3, WFQC B6, WFQC B7 and WFQC B8) were collected and analysed by SGS. Of these 6 field blanks, 4 of the blanks were found to have concentrations above the LOR. Trip Blanks: A total of 7 trip blanks (WFQC A1, WFQC A2, WFQC A3, WFQC A7, WFQC A8, WFQC A9 and WFQC A10) were collected and analysed by SGS. Of these 7 trip blanks, none of the blanks were found to have concentrations above the LOR. | 2007 Although no rinsate blanks were analysed, only disposable dedicated bailers were used to purge and sample the monitoring wells. As no trip or field blanks were analysed, potential cross contamination during transport, from equipment or due to atmospheric condition could not be determined. 2008 The following analytes for the rinsate, field and trip blank samples have reported concentrations above the laboratory detection limits: Sample Type; Sample ID; Analyte with concentration above LOR. Laboratory Report: SGS 62134. FB; WFQCB1; Carbonate Alkalinity (4.9mg/L); Chloride (0.2mg/L); Zinc (2.4ug/L).. FB; WFQCB2; Carbonate Alkalinity (4.3mg/L); Chloride (0.2mg/L); Zinc (1.9ug/L).. FB; WFQCB3; Carbonate Alkalinity (3.5mg/L); Chloride (0.1mg/L); Zinc (1.9ug/L).. RB; WFQCC1; Carbonate Alkalinity (4.4mg/L); Chloride (0.2mg/L); Zinc (2.0ug/L).. RB; WFQCC2; Carbonate Alkalinity (4.2mg/L); Chloride (0.3mg/L); Zinc (1.9ug/L).. RB; WFQCC3; Carbonate Alkalinity (4.0mg/L); Chloride (0.2mg/L); Zinc (1.8ug/L); Sulphate (0.5mg/L).. Laboratory Report: SGS 62397. FB; WFQC B8; Zinc (1.7 ug/L), Chloromethane (5.9ug/L). RB; WFQC C8; Zinc (2.6 ug/L), Chloromethane (9.0ug/L). Laboratory Report: SGS 62324. RB; WFQC C7; Napthalene (12ug/L), Acenaphthene (8.9ug/L), Benzo(a)anthracene (2.1ug/L) and Chrysene (1.7 ug/L). |
|  | A total of 2 laboratory duplicate samples had RPDs that exceeded the recommended levels. (RPDs – 30% inorganics and 50% organics) | ES0715178: RPD of 59.5% for Cadmium; ES0715041: RPD of 74.3% for Copper. In both these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerated the RPD relating to small total concentration differences. |
|  | One matrix spike result for chloride could not be determined by ALS. | The lab reported the background concentration in the analyte as too high in comparison with the spike concentration; therefore a spike recovery could not be determined. |
|  | One method blank result for TDS was found to exceed the laboratory detection limits. | A method blank outlier value of 10mg/L occurred in Amdel report 07ENCA0023305 for TDS. |
|  | The equipment supplier’s calibration certificates were not included in the report |  |
|  | Field procedures were not provided for minimisation of volatile losses. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality |
| GW 09 | No Laboratory report has been provided for QC09 (split sample) |  |
|  | Several samples were received by ALS and/or Labmark above the recommended range of 4oC + or – 2oC. In some cases, the sample temperature upon receipt by the lab was not specified on the Chain of Custody documentation. | A random check of the sample temperatures upon receipt from the lab indicated the following exceedences: Report Number; Comment. ESO901481; Temperature recorded as 9.9oC (ice bricks were present). ES0901549; Temperature recorded as 8.4oC (ice bricks were present). ES0901649; Temperature recorded as 8.7oC (ice bricks were present). ES0901762; Temperature recorded as 26.5oC (ice bricks were present). ES0901850; Temperature not specified on the COC. E041735; Temperature not specified on the COC. E041622; Temperature not specified on the COC. E041568; Temperature recorded as 7.5oC (ice bricks were present). The auditor notes that volatile losses may have occurred due to the increase in temperature. As a result, the associated VOCs may have been under-reported. |
|  | Chain of custodies were present for all samples, however, not all COCs contained the complete details for the “Relinquished by” data. As a result, this is non-compliant with the Australian Standard (AS4482.1) as the chain of responsibility of the samples between the sampler and the laboratory is broken. | The sample relinquish date was not specified on the COC for ES0901649. Although the COC did not provide an unbroken chain of custody, URS has set procedures in place for sample handling, preservation and relinquishing to laboratory couriers. As a result, the auditor believes the accountability for the integrity of the samples is still possible given this known process, and the integrity of the dataset has not been compromised. |
|  | The preservation and storage method has not been specified for the samples having a delay between sample collection and sample relinquishment. | ESO901481: Samples relinquished on 02/02/2009 and received on 03/02/2009; ES0901549: Samples relinquished on 03/02/2009 and received on 04/02/2009; ES0901762: Samples relinquished on 05/02/2009 and received on 09/02/2009; E041735: Samples relinquished on 14/02/2009 and received on 19/02/2009; E041622: Samples relinquished on 14/02/2009 and received on 19/02/2009. Although URS did not specify the preservation and storage methods employed, the URS quality assurance program for other investigations with the Stage 2a site, have included preservation and storage of samples upon collection and during transfer to the laboratory. |
|  | Sample collection dates were incorrectly entered on the COC. | For ES0901549, sample collection dates were listed on the COC ranging from 1996 to 2009. The auditor notes this discrepancy as a typing error and that all samples were collected in 2009. |
|  | The recommended holding has been exceeded for several samples within the Labmark and ALS laboratory reports. | The following holding time exceedences have been noted: Lab Report; Matrix; Analyte; Comment. E041735 (Labmark); Water; Alkalinity, Total Nitrogen, Nitrate; QC02 exceeds HT by 1 day. Ammonia; QC02 exceeds HT by 36 hours. Nitrite; QC02 exceeds HT by 2 days. E041622 (Labmark); Water; Alkalinity, TOC, Nitrate; QC13 exceeds HT by 1 day. Sulphide; QC13 exceeds HT by 1 1/2 days. E041568 (Labmark); Water; Alkalinity; QC03 exceeds HT by 1 day. Sulphide; QC03 exceeds HT by 5 1/2 days. Nitrate; QC03 exceeds HT by 6 days. Nitrite; QC03 exceeds HT by 7 days. ES0901481; Water; Ferrous Iron; MW117, MW118, MW119, MW120, MW124 exceeds HT by 1 day. NOx as N; MW124 exceeds HT by 1 day. ES0901549; Water; Ferrous Iron; MW101, MW102, MW104, MW105, MW111, MW122, MW123, MW125 exceeds HT by 1 day. ES0901762; Water; Ferrous Iron; MW09, MW10, MW109, MW110, MW48, MW49A, MW49B, MW133, QC08 exceeds HT by 3 days. Nitrite as N; MW09, MW10, MW109, MW110, MW48, MW49A, MW49B, MW133, QC08 exceeds HT by 2 days. HT: Holding Time. |
|  | One laboratory duplicate sample had an RPD that exceeded the recommended levels. (RPDs: <30% inorganics and <50% organics) | ES0901481: RPD of 84.9% for Nitratre as N The low analyte concentrations for the primary and duplicate samples exaggerated the RPD relating to small total concentration differences. |
|  | Four matrix spike results from ALS reports could not be determined. One laboratory matrix spike result exceeded the recommended range of 70% to 130%. | The matrix spike outliers were as follows: Lab Report; Sample ID; Analyte; Spike Recovery. ES0901549; MW123; Chloride; Not Determined. QC02; Chloride; Not Determined. MW104; Nitrate + Nitrite as N; 24.7%. ES0901649; MW106; Manganese; Not Determined. ES0901850; MW115; Methane; Not Determined. The four matrix spikes could not be determined as the lab reported the background concentration in the analyte as too high in comparison with the spike concentration. |
|  | RPDs for blind duplicate and split duplicate samples were not calculated within the tabulated results section of the report. Therefore, GHD calculated RPDs where there was a difference in sample result between the primary and duplicate sample to determine the duplicates in exceedence of the recommended RPD range (<30% inorganics; <50% organics). A total of 3 blinds (QC02, QC08 and QC12) and 3 splits (QC03, QC09 and QC13) had been sampled during the investigation. 13 analyte results within the blind and split samples had RPDs that exceeded the specified range. | The following analytes were found to exceed the specified RPD range: Sample ID; Analyte; Primary Result (mg/L); Duplicate Result (mg/L; RPD (%). QC12 (blind); Sulphide; 0.1; 0.2; 66.7. Cd; <0.0001; 0.0002; 6.7. QC13 (split); Ferrous Iron; 3.63; 1; 113.6. QC03 (split); Zinc; 0.008; 0.011; 31.6. QC08 (blind); Ionic Balance; 1.43; 0.49; 97.9. Methane; 1480; 824; 56.9. Ferrous Iron; 0.09; 0.06; 40.0. QC09 (split); Sulphite; <2; 4; 66.7. Iron; 0.13; 0.08; 47.6. Mercury; <0.0001; 0.002; 66.7. Ferrous Iron; 0.09; <0.5; 139.0. Sulfur; 6; 44; 152.0. In the majority of these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerated the RPDs relating to small total concentration differences. In the case of Ferrous Iron at QC13, these results exceeded the specified guideline for total iron. In addition, the laboratory limit of detection at QC09 for Ferrous Iron was higher than the Total Iron guideline for the duplicate result (Guideline: 0.2 mg/L). |
|  | Rinsate Blanks: A total of 5 rinsate blanks (QCC01, QCC04, QCC06, QCC11 and QCC14) were collected and analysed by ALS. Of these 5 rinsate blanks, 2 of the blanks (QCC11 and QCC14) were found to have low concentrations of metals detected. Field Blanks: A total of 4 field blanks (QCB01, QCB04, QCB06 and QCB10) were collected and analysed by ALS. Of these 4 field blanks, 2 of the blanks (QCB06 and QCB10) were found to have low concentrations of metals detected. | Low concentrations of metals were detected for the following rinsate blank samples: QCC11: Cadmium 0.0002 mg/L, Copper 0.014mg/L and Zinc 0.007 mg/L; QCC14: Copper 0.016mg/L and Zinc 0.01 mg/L. Low concentrations of metals were detected for the following field blank samples: QCB06: Copper 0.011mg/L and Zinc 0.006 mg/L; QCB10: Copper 0.013mg/L and Zinc 0.007 mg/L. |
|  | The equipment supplier’s calibration certificates were not included in the report |  |
|  | Field procedures were not provided for minimisation of volatile losses. | The auditor is familiar with the standard field procedures used by URS and considers them adequate to maintain sample quality |
| Pre-liminary Validation Report | A total of 17 blinds and 17 splits had been sampled during the pre-liminary validation investigation. 83 analyte results within the blind and split samples had RPDs that exceeded the specified range (<30% inorganics; <50% organics). | Of the 83 analyte results, 50 of the exceedences were associated with the metal analytical suite. RPD exceedences were noted for Arsenic, Cadmium, Chromium, Copper, Lead, Manganese, Mercury, Nickel and Zinc. The other 33 exceedences were recorded for one toluene sample, a number of PAH analytes, TPH C10-C14 ,TPH C15-C28 and TPH C29-C36. Theses exceedences can be attributed to: Low analyte concentrations which exaggerate the RPD for small total concentration differences, or Trace levels of separate phase hydrocarbons Also considering the heterogeneous nature of the materials sampled and the extensive range of RPDs calculated, the auditor considers that the exceedences are typical of what might be expected and do not preclude the use of the results for characterisation of the presence of contamination across the Waterfront Development site. |
|  | Chain of custodies were present for all samples, however, they were not correctly completed. All COCs were missing the complete details for the "Relinquished by" and "Received by" data, breaking the chain of responsibility of the samples between the sampler and the laboratory, and therefore, not complying with the Australian Standard (AS4482.1) | Although the CoCs did not provide an unbroken chain of custody, URS has set procedures in place for sample handling, preservation and relinquishing to laboratory couriers. As a result, the auditor believes the accountability for the integrity of the samples is still possible given this known process, and the integrity of the dataset has not been compromised. |
|  | Several samples were received by ALS above the recommended range of 4oC + or – 2oC. | A random check of the sample temperatures upon receipt from the lab indicated the following exceedences: Report Number; Comment. SE71353; Temperature recorded as 7oC (ice bricks were present). SE71654; Temperature recorded as 5oC (free ice was present). PE025904-R1; Temperature recorded as 16.4oC (ice present, not specified if free ice or ice bricks were present). ES0912124; Temperature recorded as 17.0oC (ice method not specified). ES0912431; Temperature recorded as 13.6oC (ice bricks were present). ES0912799; Temperature recorded as 10.6oC (ice method not specified). ES0912901; Temperature recorded as 14.5oC (ice bricks were present). ES0913069; Temperature recorded as 14.6oC (ice bricks were present). ES0914214; Temperature not recorded. ES0918474; Temperature recorded as 12.6oC (ice bricks were present). The auditor notes that volatile losses may have occurred due to the increase in temperature. As a result, the associated VOCs may have been under-reported. |
|  | The recommended holding has been exceeded for several samples. | The following holding time exceedences have been noted: Lab Report; Matrix; Analyte; Comment. SE71259; Water; PAH, TPH C10-C36; 4 samples exceed HT by 1 day. SE71654; Water; PAH, TPH C10-C36; 2 samples exceed HT by 1 day. Water; PAH, TPH C10-C36; 1 sample exceeds HT by 2 days. SE71766; Soil; Phenols; 3 samples exceed HT by 1 day. Water; PAH, TPH C10-C36; 6 samples exceed HT by 4 days. SE71766A; Soil; TPH speciation (non-volatile); 1 sample exceeds HT by 16 days. PE025904-R1; Soil; TPH C10-C36; 14 samples exceed HT by 21 days. TPH C10-C36; 1 sample exceeds HT by 22 days. PAH; 14 samples exceed HT by 22 days. PAH; 1 sample exceeds HT by 21 days. Phenols; 14 samples exceed HT by 29 days. Phenols; 1 sample exceeds HT by 21 days. ES0914214; Soil; TPH speciation (non-volatile); 1 sample exceeds HT by 15 days. HT: Holding Time. |
|  |  | For the water samples analysed for Phenols, according to the EPA publication 441, if the samples have not been acidified than the holding time is 24hrs, if acidification occurred then the holding time is 21 days. Also for analysis of TPH C6-C9 and BTEX for water samples, the holding time is 7 days if not acidified and 14 days if acidified. Sample preservation methods have not been specified, therefore the auditor has taken the most conservative approach and assumed a holding time of 24 hrs for phenols and 7 days for TPH C6-C9 and BTEX. Several holding time exceedences have occurred for these analytes. |
|  | 15 laboratory duplicate samples had results that exceeded RPDs of 30% for inorganics and 50% for organics. | Of the 15 analyte results, 14 of the exceedences were associated with the metal analytical suite. RPD exceedences were noted for Chromium, Copper, Lead, Manganese and Zinc. The other exceedence was recorded for TPH C29-C36. In these cases, the values were well below the relevant guidelines and low analyte concentrations exaggerated the RPD relating to small total concentration differences. |
|  | 11 laboratory matrix spike results exceeded the recommended range of 70% to 130%. | The following matrix recoveries exceeded the recommended range, with all results below the lower limit of 70%: Lab Report; Analyte; Spike Recovery. SE71259; Benzene; 69%. Toluene; 67%. Ethylbenzene; 64%. Total Xylene; 68%. SE71699; Benzene; 66%. Ethylbenzene; 63%. Total Xylene; 64%. TRH C6 - C9; 60%. ES0912431; 2-Nitrophenol; 66.20%. ES0913069; Pentachloro-phenol; 47.50%. ES0918474; Pentachloro-phenol; 60.80%. The spike recoveries reported for the SGS analytical laboratory reports (SE prefix) were only slightly below the lower limit of the recommended range for matrix spikes. Although the spike recoveries for the ALS reports (ESO prefix) exceed the recommended range, the results are still within the recovery limits assigned by ALS for those particular analytes. |
|  | No testpit logs have been included with the report | URS notes in the report, "the testpits will be provided at a later date". These testpit logs are included in the final report. |

Notes

1. Refer also to the text following the Table for further commentary on significance.

2. Occurred when sampling occurred over a time period, ie days-months.